Smart Security System

with Laser using Logic Gates

Digital Logic Design Project Report

Using 7400-series TTL ICs (7404, 7408 and 7432)

Submitted by

Muneeb Ur Rehman Sidhar

Department of Computer Science

University of Engineering and Technology (UET), Taxila

ACKNOWLEDGEMENT

All praise belongs to Almighty Allah, whose blessings and guidance made this work possible. We send Durood upon the Holy Prophet Hazrat Muhammad (PBUH), who showed humanity the path of knowledge and righteousness.

We wish to express our sincere gratitude to our course instructor, [Instructor Name], for his patient guidance, constructive feedback, and encouragement throughout the duration of this project. His teaching of Digital Logic Design concepts gave us the foundation and confidence to undertake this work.

We are deeply thankful to our parents for their unconditional love, prayers, and support at every stage of our academic journey. We would also like to thank our colleagues and friends whose assistance and motivation were invaluable during this project.

ABSTRACT

This report presents the design and physical implementation of a Smart Security System with Laser using combinational digital logic integrated circuits. The system was built using 7400-series TTL ICs, specifically the 7404 Hex NOT Gate, 7408 Quad AND Gate, and 7432 Quad OR Gate, without the use of any microcontroller or software programming.

A laser diode module projects a narrow beam of light onto a light-dependent resistor (LDR) sensor module placed across a protected entrance. As long as the beam reaches the sensor, the system remains in its normal state. When a person or object interrupts the beam, the sensor output changes and the logic circuit activates an alarm made of a buzzer and a warning LED. A hardware latch, created using AND-OR gate feedback, keeps the alarm ON even after the beam is restored, so that an intruder cannot silence the alarm by simply moving away. An arm switch enables or disables the system, and a reset push button clears the alarm.

The primary objective of this project is to provide a simple, low-cost and reliable security solution for protecting doors, cabinets, lockers and other restricted areas. The project was assembled on a breadboard and tested, confirming that the circuit behaves as designed. It demonstrates a practical application of Digital Logic Design to a real-world security problem.

TABLE OF CONTENTS

| ACKNOWLEDGEMENT | 2 |
| --- | --- |
| ABSTRACT | 3 |
| TABLE OF CONTENTS | 4 |
| 1. INTRODUCTION | 5 |
| 2. PROJECT AIM AND OBJECTIVES | 6 |
| 3. DESCRIPTION OF THE PROJECT | 7 |
| 4. COMPONENTS REQUIRED | 8 |
| 5. DESCRIPTION OF COMPONENTS | 9 |
| 6. CIRCUIT CONNECTIONS | 11 |
| 7. WORKING PRINCIPLE | 13 |
| 8. TRUTH TABLE | 14 |
| 9. PROS AND CONS | 15 |
| 10. APPLICATIONS | 16 |
| 11. FUTURE DEVELOPMENT | 16 |
| 12. CONCLUSION | 16 |

1. INTRODUCTION

Security is a basic requirement in homes, offices, shops, laboratories and educational institutions. Valuable equipment, documents and money are often kept in rooms or cabinets that must be protected from unauthorized access. Commercial security systems are available, but many of them are expensive, complicated to install, and depend on microcontrollers or proprietary software that ordinary users cannot easily understand or repair.

The Smart Security System with Laser is a proposed solution to this problem. It is a digital logic-based circuit that raises an alarm when an invisible laser beam across an entrance is broken. A laser diode module is aligned with an LDR sensor module on the opposite side of the protected path. While the beam is received, the sensor reports a normal condition. When the beam is interrupted, the sensor reports an intrusion, and the signal is processed by a combination of NOT, AND and OR logic gates, which drives a buzzer and an LED.

What makes this project particularly interesting is that it achieves a latching, switchable alarm without using any microcontroller or software. The entire logic is implemented using only three types of 7400-series TTL integrated circuits: the 7404, the 7408 and the 7432. A hardware latch built from AND-OR gate feedback holds the alarm ON after the intrusion, in the same way an SR latch stores a state, but using only basic combinational gates.

This project was designed and implemented as part of the Digital Logic Design course. It serves both an academic and a practical purpose, demonstrating how fundamental logic gate theory can be applied to build a working security device.

2. PROJECT AIM AND OBJECTIVES

Project Aim

The aim of this project is to design and implement a Smart Security System with Laser using digital logic gate ICs that can detect an intrusion across a protected path and raise a latched audible and visual alarm, without requiring any microcontroller or software programming.

Objectives

- To design a digital circuit using 7400-series TTL ICs that automatically sounds an alarm when a laser beam is interrupted.

- To implement a hardware latch using AND and OR gate feedback so that the alarm stays ON even after the beam is restored.

- To provide an arm switch so that the system can be enabled or disabled by an authorized person.

- To provide a reset push button that clears the latched alarm and returns the system to its waiting state.

- To understand and apply the practical working of NOT, AND and OR gates in a real hardware circuit.

- To study the behavior of a laser module and an LDR sensor module and integrate them with TTL logic gate ICs on a breadboard.

- To demonstrate a simple, reliable and low-cost security solution built entirely from discrete digital components.

3. DESCRIPTION OF THE PROJECT

The Smart Security System with Laser is a hardware-only intrusion alarm. A laser diode module is placed on one side of the protected path, and an LDR sensor module is placed on the opposite side, exactly in line with the laser beam. The sensor module has a digital output (DO) pin. While the laser light falls on the LDR, the output stays LOW, which represents a normal condition. When the beam is blocked, the LDR receives only ambient light, and the output of the module goes HIGH, which represents an intrusion.

The digital logic circuit processes the sensor signal together with two control inputs: an ARM switch and a RESET push button. The first AND gate (7408) combines the intrusion signal with the ARM switch, so that the alarm can only be triggered while the system is armed. The output of this gate is called the trigger signal. The OR gate (7432) combines the trigger signal with the feedback signal coming from the final output. The NOT gate (7404) inverts the signal of the RESET button, and the second AND gate (7408) combines the OR gate output with this inverted reset signal to produce the final alarm output Q.

The most important feature of this circuit is its feedback or latch mechanism. Once the trigger signal goes HIGH and the output Q goes HIGH, Q is fed back into the second input of the OR gate. This creates a self-sustaining loop: even after the laser beam is restored and the trigger signal returns to LOW, the OR gate output stays HIGH because it is held by the feedback from Q. The alarm therefore keeps sounding until it is deliberately reset.

The alarm turns OFF only when the RESET button is pressed. At that moment, the NOT gate output goes LOW and forces the second AND gate output LOW, which breaks the feedback loop. The circuit then returns to its waiting state, ready to detect the next intrusion. The output Q drives a red LED directly through a resistor, and drives the buzzer through an NPN transistor, because a TTL gate output cannot supply enough current for a buzzer.

4. COMPONENTS REQUIRED

The following components were used to assemble the Smart Security System on a breadboard:

| Component | Quantity | Purpose |
| --- | --- | --- |
| IC 7404 - Hex NOT Gate | 1 | Inverts the RESET button signal |
| IC 7408 - Quad AND Gate | 1 | Forms the trigger signal and the final alarm output |
| IC 7432 - Quad OR Gate | 1 | Combines the trigger signal with feedback for latching |
| Laser Diode Module (5V) | 1 | Produces the laser beam across the protected path |
| LDR Sensor Module | 1 | Detects whether the laser beam is received or blocked |
| NPN Transistor (BC547) | 1 | Drives the buzzer from the logic output |
| Active Buzzer (5V) | 1 | Audible alarm |
| LED (Red) | 1 | Visual alarm indicator |
| Resistor 330 Ohm | 1 | Current-limiting protection for the LED |
| Resistor 1 kOhm | 1 | Base resistor for the transistor |
| Resistor 10 kOhm | 2 | Pull-down resistors for the ARM switch and RESET button |
| Slide Switch | 1 | ARM / DISARM control |
| Push Button | 1 | RESET control |
| Breadboard | 1 | Circuit assembly and prototyping platform |
| Connecting Wires | Several | Signal and power connections between components |
| 5V DC Power Supply | 1 | Powers all ICs and modules |

5. DESCRIPTION OF COMPONENTS

IC 7404 Hex Inverter (NOT Gate)

The 7404 is a standard TTL integrated circuit containing six independent single-input NOT gates. Each gate produces the logical complement of its input: if the input is LOW (logic 0), the output is HIGH (logic 1), and vice versa. In this circuit, only one gate of the 7404 is used. It inverts the signal of the RESET button so that pressing the button produces a LOW signal that can force the output AND gate LOW. Pin 14 is connected to +5V and Pin 7 to GND to power the IC.

IC 7408 Quad 2-Input AND Gate

The 7408 is a TTL IC containing four independent 2-input AND gates. The output of each AND gate is HIGH only when both of its inputs are HIGH at the same time. If either input is LOW, the output is LOW. In this design, two of the four gates are used. The first gate combines the intrusion signal with the ARM switch to produce the trigger signal. The second gate combines the OR gate output with the inverted reset signal and produces the final alarm output Q. Pin 14 is connected to +5V and Pin 7 to GND.

IC 7432 Quad 2-Input OR Gate

The 7432 is a TTL IC containing four independent 2-input OR gates. Its output is HIGH when either or both of its inputs are HIGH. The output is LOW only when both inputs are LOW. In this circuit, one OR gate forms the core of the latch by combining the trigger signal with the feedback from the alarm output. Once either input goes HIGH, the OR gate output goes HIGH, and through the latch loop it keeps itself HIGH. Pin 14 is connected to +5V and Pin 7 to GND.

Laser Diode Module

The laser diode module is a small red laser (typically 650 nm wavelength and less than 5 mW output) mounted on a small board that operates at 5V. It produces a narrow, straight and highly visible beam that can travel over a long distance without spreading. This makes it ideal for creating a tripwire-style security beam. The module has a signal pin, a power pin and a ground pin. In this project it is connected permanently to +5V, so that the beam is always ON while the system is powered. Care should be taken never to point the laser at anyone's eyes.

LDR Sensor Module

The LDR (light-dependent resistor) sensor module contains an LDR, an LM393 comparator and an adjustable potentiometer. The resistance of the LDR drops when light falls on it and rises in darkness. The comparator compares this change with a threshold set by the potentiometer and gives a clean digital signal at the DO (digital output) pin. For the module used in this project, the DO pin stays LOW while the laser light is falling on the LDR and goes HIGH when the beam is blocked. The potentiometer is adjusted until the module reacts reliably to the laser but not to ordinary room light. The module operates on 3.3V to 5V.

NPN Transistor (BC547) and Buzzer

A standard TTL gate can supply only a very small output current, which is not enough to sound a buzzer. For this reason a BC547 NPN transistor is used as an electronic switch. The alarm output Q is connected to the transistor base through a 1 kOhm resistor. When Q is HIGH, the transistor conducts and the active buzzer, which is connected between +5V and the collector, sounds. When Q is LOW, the transistor is OFF and the buzzer is silent. The emitter is connected to GND.

LED and Resistors

A standard 5mm red LED is used as a visual alarm indicator. It is driven from the alarm output Q through a 330 ohm current-limiting resistor, which prevents excess current from damaging the LED and the output pin of the IC. The anode of the LED connects to the resistor and the cathode connects to GND. Two 10 kOhm pull-down resistors are used on the ARM switch and the RESET button, so that these inputs always have a defined LOW level when the switch is open and the button is not pressed.

6. CIRCUIT CONNECTIONS

Step 1: Power All ICs and Modules

Before making any signal connections, all three ICs must be supplied with power. For the 7404, 7408 and 7432, Pin 14 of each IC is connected to the +5V supply rail on the breadboard and Pin 7 of each IC is connected to the GND rail. The laser diode module and the LDR sensor module are also connected to +5V and GND. Any unused gate inputs of the ICs are connected to GND so that they do not float.

Step 2: Intrusion Signal and Arm Switch (Trigger Gate)

The DO pin of the LDR sensor module gives a HIGH signal when the laser beam is blocked. This signal is combined with the ARM switch in the first AND gate:

- The DO pin of the LDR sensor module is connected to Pin 1 of the 7408 IC (first AND gate, input A).

- One side of the ARM slide switch is connected to +5V and the other side to Pin 2 of the 7408 IC (first AND gate, input B). A 10 kOhm pull-down resistor connects Pin 2 to GND.

Result: Pin 3 of the 7408 (the trigger signal) goes HIGH only when the beam is blocked and the ARM switch is ON.

Step 3: Trigger Signal to the OR Gate

- Pin 3 of the 7408 IC (trigger signal) is connected to Pin 1 of the 7432 IC (OR gate, input A).

Step 4: Feedback Memory (The Latch)

To keep the alarm ON after the beam is restored, the final alarm output is fed back to the OR gate:

- Pin 6 of the 7408 IC (second AND gate output, Q) is connected to Pin 2 of the 7432 IC (OR gate, input B).

Once Q goes HIGH, this feedback holds the OR gate output HIGH continuously, regardless of whether the laser beam is still blocked or not. This is the latch mechanism of the circuit.

Step 5: Reset Button Through the NOT Gate

The RESET push button is connected so that pressing it produces a HIGH level, which is then inverted by the NOT gate:

- One side of the RESET push button is connected to +5V and the other side to Pin 1 of the 7404 IC (NOT gate input). A 10 kOhm pull-down resistor connects Pin 1 to GND.

- Pin 2 of the 7404 IC (NOT gate output) is connected to Pin 5 of the 7408 IC (second AND gate, input B).

Under normal conditions, the button is not pressed, Pin 1 of the 7404 is LOW, and its output is HIGH, so the second AND gate is ready to pass signals. When the button is pressed, the NOT gate output goes LOW and forces the second AND gate output LOW.

Step 6: OR Gate Output to the Output AND Gate

- Pin 3 of the 7432 IC (OR gate output) is connected to Pin 4 of the 7408 IC (second AND gate, input A).

Step 7: LED and Buzzer Output

The alarm output Q (Pin 6 of the 7408) drives both the LED and the buzzer:

- Pin 6 of the 7408 IC is connected to one end of the 330 ohm resistor. The other end of the resistor is connected to the anode of the LED, and the cathode of the LED is connected to GND.

- Pin 6 of the 7408 IC is also connected to one end of the 1 kOhm resistor. The other end of the resistor is connected to the base of the BC547 transistor.

- The emitter of the BC547 is connected to GND. The negative terminal of the buzzer is connected to the collector, and the positive terminal of the buzzer is connected to +5V.

Step 8: Aligning the Laser

The laser diode module is fixed on one side of the protected path, and the LDR sensor module is fixed on the opposite side, so that the beam falls directly on the LDR. The potentiometer on the sensor module is then adjusted until the DO output stays LOW while the beam is received and goes HIGH immediately when the beam is blocked by a hand or object. If a different sensor module with the opposite output polarity is used, one extra NOT gate should be placed between the DO pin and Pin 1 of the 7408.

7. WORKING PRINCIPLE

Initial State

When the circuit is first powered ON with the ARM switch ON, the laser beam is falling on the LDR. The DO output of the sensor module is LOW, so the trigger signal is LOW. The feedback from Q is also LOW initially. With both OR inputs LOW, the OR output is LOW, the output AND gate gives LOW, and the LED and buzzer are OFF. The system waits quietly for the beam to be interrupted.

When the Beam Is Broken - Alarm Turns ON

When a person or object passes through the laser beam, the LDR no longer receives the laser light. The DO output of the sensor module goes HIGH. Because the ARM switch is ON, the first AND gate produces a HIGH trigger signal. The OR gate receives HIGH at input A and immediately produces a HIGH output. The second AND gate has HIGH at input A, and since the reset path is HIGH (RESET not pressed), its output Q goes HIGH. The LED lights up and the transistor switches the buzzer ON. At the same moment, the HIGH Q feeds back into OR gate input B, locking the latch in the ON state.

Alarm Stays ON - The Latch Holds

After the intruder has passed and the laser beam falls on the LDR again, the DO output returns to LOW and the trigger signal drops to LOW. However, OR gate input B is still being held HIGH by the feedback from Q. Since at least one OR input is HIGH, the OR output remains HIGH, the output AND gate continues to give HIGH, and the alarm keeps sounding. The latch has stored the alarm state without any flip-flop or microcontroller memory.

When RESET Is Pressed - Alarm Turns OFF

When the authorized person presses the RESET button, a HIGH level reaches the NOT gate, and its output becomes LOW. This LOW arrives at input B of the second AND gate and immediately pulls Q to LOW. The LED turns OFF and the buzzer stops. With Q now LOW, the feedback into the OR gate disappears. If the laser beam is falling on the LDR, both OR inputs are LOW and the OR output drops to LOW, so the circuit returns to its idle state, ready to detect the next intrusion.

Disarmed Condition

When the ARM switch is turned OFF, input B of the first AND gate is LOW, so the trigger signal always stays LOW, no matter whether the beam is blocked or not. The system therefore does not sound the alarm, which allows authorized people to pass through the protected path safely.

8. TRUTH TABLE

The table below shows the logic states at key points in the circuit for each operating condition. I is the DO output of the sensor module (1 means the beam is blocked). ARM is the state of the arm switch (1 means armed). RESET is the state of the reset button (1 means pressed). Previous Q is the alarm output before the new input is applied, Trigger is the output of the first AND gate, and New Q is the alarm output after the inputs are applied.

| I | ARM | RESET | Previous Q | Trigger | New Q | System State |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 0 | 0 | 0 | Normal (beam received) |
| 1 | 0 | 0 | 0 | 0 | 0 | Disarmed (no alarm) |
| 1 | 1 | 0 | 0 | 1 | 1 | Alarm ON (intrusion) |
| 0 | 1 | 0 | 1 | 0 | 1 | Alarm latched |
| 0 | 1 | 1 | 1 | 0 | 0 | Reset (alarm OFF) |

Note: In the fourth row, even though the beam has been restored (I = 0) and the trigger signal is LOW, the feedback from Q keeps the OR output HIGH, which demonstrates the latch behavior. The alarm turns OFF only when the RESET button is pressed (RESET = 1), as shown in the last row.

9. PROS AND CONS

Advantages

- The system requires no microcontroller or any form of software programming, making it extremely simple to understand and replicate.

- It uses standard 7400-series TTL ICs that are widely available and very affordable, keeping the overall cost of the project low.

- The circuit is entirely hardware-based, which means it is reliable and not affected by software crashes, bugs or boot delays.

- The latching alarm cannot be silenced by an intruder simply moving out of the beam; only the RESET button stops it.

- The arm switch lets authorized people disable the system when needed.

- The laser beam is narrow and works over a long distance, so a single beam can protect a long corridor or door.

- The project is an excellent practical demonstration of how NOT, AND and OR gates work together in a real circuit.

Disadvantages

- A single laser beam protects only one line. An intruder who steps over or around the beam will not be detected.

- The laser and the LDR sensor must be carefully aligned. Vibrations or movement of the modules can cause false alarms.

- Strong ambient light, such as direct sunlight, can affect the LDR sensor and reduce its reliability.

- Because the RESET button is on the same circuit, anyone who finds it can silence the alarm. A hidden or key-operated switch would be safer.

- The circuit has no way to report which beam was broken or how many times, and it does not send any remote notification.

- Scaling this design to several beams would require more ICs, more sensors and more connections.

10. APPLICATIONS

- Protecting doors, windows and entrances of homes and small shops against unauthorized entry.

- Securing cabinets, lockers, safes and display counters where valuable items are stored.

- Protecting laboratories and computer rooms in educational institutions.

- Guarding the boundary of a restricted area, corridor or staircase.

- Use as a tripwire alarm in warehouses and storage rooms.

- A teaching model for Digital Logic Design laboratory sessions, illustrating latch behavior, sensor interfacing and gate-level circuit design.

- A foundational prototype that can be extended into a more complete smart security system with additional sensors.

11. FUTURE DEVELOPMENT

- Multiple laser beams and mirrors can be used to create a grid or zig-zag pattern, so that the protected area cannot be crossed without breaking a beam.

- A 555 timer IC can be added so that the buzzer sounds for a fixed time and then switches OFF automatically, and also to add an entry delay after arming.

- A keypad or key switch based reset can replace the push button, so that only authorized people can silence the alarm.

- A modulated laser signal (for example, generated by a 555 timer) can be used with a matching receiver so that ambient light and flashlights cannot fool the system.

- The system could be extended with a GSM module or a Wi-Fi board to send an SMS or app notification to the owner when the alarm is triggered.

- In a more advanced version, the circuit could be replaced with an Arduino to log intrusion events and control additional devices such as a siren and a camera.

12. CONCLUSION

The Smart Security System with Laser is a simple yet effective project that successfully demonstrates the practical application of digital logic gates in solving a real-world security problem. Using only three types of standard TTL ICs, the 7404, 7408 and 7432, along with a laser diode module and an LDR sensor module, the circuit achieves an automatic, latching and switchable alarm entirely through hardware logic, without any software or microcontroller involvement.

The circuit was assembled on a breadboard and tested. The results confirmed that breaking the laser beam triggers the alarm, the AND-OR feedback latch keeps the alarm ON after the beam is restored, the ARM switch can disable the system, and the RESET button correctly returns the circuit to its waiting state. The circuit performed as expected under the tested conditions.

Through this project, we gained valuable hands-on experience with IC pin configurations, 7400-series TTL logic, signal flow through combinational gates, transistor switching, and the concept of hardware latching. This project has laid a strong foundation for exploring more advanced topics such as sequential logic, sensor integration and embedded systems in future coursework.